Technical Specification for XD05PRO Amplifier Module

We release the full technical documentation of the XD05PRO amplifier module to encourage third-party developers to develop high-performance custom amplifier modules for the XD05PRO. All electronics enthusiasts and developers are welcome to join the customization development.

This amplifier module only consists of the operational amplifier (OP-AMP) stage. The rear-stage current buffer circuit is a Class-A current expansion circuit built with bipolar transistors, which is integrated on the main unit motherboard.

The stock factory amplifier module is constructed with two operational amplifier ICs and an RC feedback network. Each op-amp separately handles the positive-phase and negative-phase signal paths for the balanced topology of the left and right channels.

The device features three gain modes: Eco, Normal and Turbo, which supply the op-amps with operating voltages of ±6.5V, ±10V and ±15V respectively. During circuit design of the amplifier module, the rated withstand voltage of all components must exceed ±15V.

Strictly control the DC offset voltage at the output of the amplifier circuit during the design phase. Excessive DC offset will trigger loud popping noise during power-on and power-off.

Two files are attached: one is the schematic diagram of the amplifier module, and the other is the PCB project file. The PCB project is designed with Altium Designer. Please convert the file format by yourself if you need to open it with other PCB design software.